ONVIF™

Imaging Client Test Specification

Version 16.07

July 2016

© 2016 ONVIF, Inc. All rights reserved. www.onvif.org

Recipients of this document may copy, distribute, publish, or display this document so long as this copyright notice, license and disclaimer are retained with all copies of the document. No license is granted to modify this document.

THIS DOCUMENT IS PROVIDED "AS IS," AND THE CORPORATION AND

ITS MEMBERS AND THEIR AFFILIATES, MAKE NO REPRESENTATIONS

OR WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED

TO, WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR

PURPOSE, NON-INFRINGEMENT, OR TITLE; THAT THE CONTENTS OF

THIS DOCUMENT ARE SUITABLE FOR ANY PURPOSE; OR THAT THE IMPLEMENTATION OF SUCH CONTENTS WILL NOT INFRINGE ANY PATENTS, COPYRIGHTS, TRADEMARKS OR OTHER RIGHTS.

IN NO EVENT WILL THE CORPORATION OR ITS MEMBERS OR THEIR

AFFILIATES BE LIABLE FOR ANY DIRECT, INDIRECT, SPECIAL,

INCIDENTAL, PUNITIVE OR CONSEQUENTIAL DAMAGES, ARISING OUT

OF OR RELATING TO ANY USE OR DISTRIBUTION OF THIS DOCUMENT,

WHETHER OR NOT (1) THE CORPORATION, MEMBERS OR THEIR AFFILIATES HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES, OR (2) SUCH DAMAGES WERE REASONABLY FORESEEABLE,

AND ARISING OUT OF OR RELATING TO ANY USE OR DISTRIBUTION OF

THIS DOCUMENT. THE FOREGOING DISCLAIMER AND LIMITATION ON

LIABILITY DO NOT APPLY TO, INVALIDATE, OR LIMIT REPRESENTATIONS

AND WARRANTIES MADE BY THE MEMBERS AND THEIR RESPECTIVE AFFILIATES TO THE CORPORATION AND OTHER MEMBERS IN CERTAIN WRITTEN POLICIES OF THE CORPORATION.

Table of Contents

1. Introduction	4

1.1. Scope	4

1.2. Get Imaging Capabilities	4

1.3. Get Imaging Settings	4

1.4. Imaging Settings Configuration	4

1.5. Focus Move Capabilities	5

1.6. Focus Control	5

2. Normative references	6

3. Terms and Definitions	7

3.1. Conventions	7

3.2. Definitions	7

3.3. Abbreviations	8

3.4. Namespaces	8

4. Test Overview	10

4.1. Test Setup	10

4.2. Prerequisites	10

5. Get Imaging Capabilities Test Cases	11

5.1. Expected Scenarios Under Test:	11

5.2. GET CAPABILITIES	11

5.3. GET SERVICE CAPABILITIES	12

6. Get Imaging Settings Test Cases	14

6.1. Expected Scenarios Under Test:	14

6.2. GET IMAGING SETTINGS	14

7. Imaging Settings Configuration Test Cases	16

7.1. Expected Scenarios Under Test:	16

7.2. GET OPTIONS	16

7.3. SET IMAGING SETTINGS	17

8. Focus Move Capabilities Test Cases	24

8.1. Expected Scenarios Under Test:	24

8.2. GET FOCUS MOVE OPTIONS	24

9. Focus Control Test Cases	26

9.1. Expected Scenarios Under Test:	26

9.2. ABSOLUTE FOCUS MOVE	26

9.3. RELATIVE FOCUS MOVE	28

9.4. CONTINUOUS FOCUS MOVE	29

9.5. STOP	31

A. Revision History	32

Introduction

The goal of the ONVIF Test Specification set is to make it possible to realize fully interoperable IP physical security implementations from different vendors. This specification also acts as an input document to the development of a test tool which will be used to test the ONVIF Client implementation conformance towards ONVIF standard. This Client Test Tool analyzes network communications between ONVIF Devices and Clients being tested and determines whether a specific Client is ONVIF conformant (see ONVIF Conformance Process Specification [http:// www.onvif.org/Documents/Specifications.aspx]).

This particular document defines test cases required for testing Imaging Service features of a Client application e.g. Get Imaging Capabilities, Video Sources List, Get Imaging Settings, Imaging Settings Configuration, Focus Control. It also describes the test framework, test setup, prerequisites, test policies needed for the execution of the described test cases.

Scope

This ONVIF Imaging Client Test Specification defines and regulates the conformance testing procedure for the ONVIF conformant Clients in the scope of Imaging Service features. Conformance testing is meant to be black-box network traces analysis and verification. The objective of this specification is to provide the test cases to test individual requirements of ONVIF Clients in the scope of Imaging Service features according to ONVIF Imaging Service Specification.

The principal intended purposes are:

Imaging Settings Configuration section specifies Client ability to change imaging settings on

Device.

Provide self-assessment tool for implementations.

- Provide comprehensive test suite coverage for Imaging Service features.

This specification does not address the following:

- 3rd parties Client use cases

- Non-functional (performance and regression) testing and analysis.

- SOAP Implementation Interoperability test i.e. Web Services Interoperability Basic Profile version

2.0 (WS-I BP2.0).

- Network protocol implementation Conformance test for HTTPS and HTTP protocols.

The following sections cover test cases needed for the verification of relevant features as mentioned in the ONVIF Profile Specifications.

Get Imaging Capabilities

Get Imaging Capabilities section specifies Client ability to request imaging capabilities from Device.

Get Imaging Settings

Get Imaging Settings section specifies Client ability to request imaging settings from Device.

Imaging Settings Configuration

Focus Move Capabilities

Focus Move Capabilities section specifies Client ability to retrieve focus move capabilities from Device.

Focus Control

Focus Control section specifies Client ability to control focus on Device.

Normative references

ONVIF Conformance Process Specification:

http://www.onvif.org/Documents/Specifications.aspx ONVIF Profile Policy: http://www.onvif.org/Documents/Specifications.aspx ONVIF Core Specifications: http://www.onvif.org/Documents/Specifications.aspx ONVIF Core Client Test Specification:

http://www.onvif.org/Documents/Specifications.aspx ONVIF Imaging Specification:

http://www.onvif.org/Documents/Specifications.aspx

ISO/IEC Directives, Part 2: http://www.iso.org/directives

ISO 16484-5:2014-09 Annex P:

https://www.iso.org/obp/ui/#!iso:std:63753:en W3C SOAP 1.2, Part 1, Messaging Framework:

http://www.w3.org/TR/soap12-part1/

W3C XML Schema Part 1: Structures Second Edition:

http://www.w3.org/TR/xmlschema-1/

W3C XML Schema Part 2: Datatypes Second Edition:

"http://www.w3.org/TR/xmlschema-2/ [http://www.w3.org/TR/xmlschema-2/]

Rules for the structure and drafting of International Standards, Annex H: Verbal forms for the expression of provisions.

Terms and Definitions

Conventions

The key words "shall", "shall not", "should", "should not", "may", "need not", "can", "cannot" in this specification are to be interpreted as described in [ISO/IEC Directives Part 2].

Definitions

This section describes terms and definitions used in this document.

Profile

See ONVIF Profile Policy.

ONVIF Device

Computer appliance or software program that exposes one or multiple ONVIF Web Services.

ONVIF Client

Computer appliance or software program that uses ONVIF Web Services.

Image Stabilization

Functionality used to avoid blurring of images due to movement of the device or its objects.

Conversation

A Conversation is all exchanges between two MAC addresses that contains SOAP request and response.

Network

A network is an interconnected group of devices communicating using the Internet protocol.

Network Trace Capture file

Data file created by a network protocol analyzer software (such as Wireshark). Contains network packets data recorded during a live network communications.

SOAP

SOAP is a lightweight protocol intended for exchanging structured information in a decentralized, distributed environment. It uses XML technologies to define an extensible messaging framework providing a message construct that can be exchanged over a variety of underlying protocols.

Client Test Tool

ONVIF Client Test Tool that tests ONVIF Client implementation towards the ONVIF Test Specification set.

Imaging Service

Services for exposure time, gain and white balance parameters among others.

Tone Compensation

Functionality used to make the image with dark or bright areas to be more visible.

Defogging

Functionality used to make the image more detailed in presence of fog.

Valid Device Response

Device has responded to specific request with code HTTP or RTSP 200 OK and SOAP fault message has not appeared.

Abbreviations

This section describes abbreviations used in this document.

HTTP

Hyper Text Transport Protocol.

HTTPS

Hyper Text Transport Protocol over Secure Socket Layer.

URI

Uniform Resource Identifier.

WSDL

Web Services Description Language.

XML

eXtensible Markup Language.

Namespaces

Prefix and namespaces used in this test specification are listed in Table 1. These prefixes are not part of the standard and an implementation can use any prefix.

Table 3.1. Defined namespaces in this specification

| Prefix | Namespace URI | Description |
| --- | --- | --- |
| soapenv | http://www.w3.org/2003/05/soapenvelope | Envelope namespace as defined by SOAP 1.2 [SOAP 1.2, Part 1] |
| xs | http://www.w3.org/2001/XMLSchema | Instance namespace as defined by XS [XML- Schema, Part1] and [XMLSchema,Part 2] |
| xsi | http://www.w3.org/2001/XMLSchemainstance | XML schema instance namespace |
| tns1 | http://www.onvif.org/ver10/topics | The namespace for the ONVIF topic namespace |
| tt | http://www.onvif.org/ver10/schema | ONVIF XML schema descriptions |
| tds | http://www.onvif.org/ver10/device/ wsdl | The namespace for the WSDL device service |

| Prefix | Namespace URI | Description |
| --- | --- | --- |
| trt | http://www.onvif.org/ver10/media/wsdl | The namespace for the WSDL media service |
| timg | http://www.onvif.org/ver20/imaging/ wsd | The namespace for the WSDL imaging service |

Test Overview

This section provides information for the test setup procedure and required prerequisites that should be followed during test case execution.

An ONVIF client with Imaging features support can provide image settings configuration and focus control.

An ONVIF Profile is described by a fixed set of functionalities through a number of services that are provided by the ONVIF standard. A number of services and functionalities are mandatory for each type of ONVIF Profile. An ONVIF Device and ONVIF Client may support any combination of Profiles and other optional services and functionalities.

Test Setup

Collect Network Traces files required by the test cases.

Collect Feature List XML files for Devices detected in the Network Trace files.

Client shall support all mandatory and conditional features listed in the Device Feature List XML file supplied for the Profiles supported by the Client.

For ONVIF compatibility, the ONVIF Client shall follow the requirements of the conformance process. For details please see the latest ONVIF Conformance Process Specification.

Prerequisites

The pre-requisites for executing the test cases described in this Test Specification include:

The Device shall be configured with an IPv4 address.

The Device shall be able to be discovered by the Client.

Get Imaging Capabilities Test Cases

Validated Feature: get_imaging_capabilities

Profile A Requirement: None

Profile C Requirement: None

Profile G Requirement: None

Profile Q Requirement: None

Profile S Requirement: None

Expected Scenarios Under Test:

- Client connects to Device to retrieve a imaging capabilities.

- Client is considered as supporting Get Imaging Capabilities if the following conditions are met:

  - Client 	is 	able 	to 	retrieve 	a 	imaging 	capabilities 	using 	GetCapabilities operation OR GetServiceCapabilities operation (Imaging Service) OR supports get_services_capabilities.get_services feature.

- Client is considered as NOT supporting Get Imaging Capabilities if ANY of the following is TRUE:

  - No valid responses for GetCapabilities request if detected AND Device supportes GetCapabilities feature OR

  - No valid responses for GetServiceCapabilities request (Imaging Service) if detected AND Device supportes GetServices feature

  - No valid responses for GetCapabilities request AND no valid responses for GetServiceCapabilities request (Imaging Service) AND get_services_capabilities.get_services feature is not supported by Client.

GET CAPABILITIES

Test Label: Get Imaging Capabilities - Get Capabilities

Test Case ID: GETIMAGINGCAPABILITIES-1

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Get Capabilities

Test Purpose: To verify that imaging capabilities provided by Device is received by Client using the GetCapabilities operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with GetCapabilities operation with tds:Category element equal to "All" OR "Imaging" OR without any tds:Category element present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetCapabilities request message with tds:Category element equal to "All" OR "Imaging" OR without any tds:Category element to retrieve imaging capabilities from the Device.

- Device responses with code HTTP 200 OK and GetCapabilitiesResponse message.

Test Result:

PASS -

- Client GetCapabilities request messages are valid according to XML Schemas listed in Namespaces AND

- Client GetCapabilities request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element tds:GetCapabilities AND

- [S2] IF it contains any tds:Category element THEN it contains tds:Category element equal to "All" OR "Imaging" AND

- Device response on the GetCapabilities request fulfills the following requirements:

the

GetServiceCapabilities

operation.

Pre-Requisite:

[S3] It has HTTP 200 response code AND

- [S4] soapenv:Body element has child element tds:GetCapabilitiesResponse.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_imaging_capabilities.get_capabilities

GET SERVICE CAPABILITIES

Test Label: Get Imaging Capabilities - Get Service Capabilities

Test Case ID: GETIMAGINGCAPABILITIES-2

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Get Services

Test Purpose: To verify that imaging capabilities provided by Device is received by Client using

- The Network Trace Capture files contains at least one Conversation between Client and Device with GetServiceCapabilities operation for Imaging Service present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetServiceCapabilities request message to retrieve imaging capabilities from the Device.

- Device responses with code HTTP 200 OK and GetServiceCapabilitiesResponse message.

Test Result:

PASS -

- Client GetServiceCapabilities request messages are valid according to XML Schemas listed in Namespaces AND

- Client GetServiceCapabilities request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:GetServiceCapabilities AND

- Device response on the GetServiceCapabilities request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:GetServiceCapabilitiesResponse.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_imaging_capabilities.get_service_capabilities

Get Imaging Settings Test Cases

Validated Feature: get_imaging_settings

Profile A Requirement: None

Profile C Requirement: None

Profile G Requirement: None

Profile Q Requirement: None

Profile S Requirement: None

Expected Scenarios Under Test:

- Client connects to Device to retrieve a current imaging settings.

- Client is considered as supporting Get Imaging Settings if the following conditions are met:

  - Client is able to retrieve a current imaging settings using GetImagingSettings operation.

- Client is considered as NOT supporting Get Imaging Settings if ANY of the following is TRUE:

  - No valid responses for GetImagingSettings request.

1.

Client invokes

GetImagingSettings

request message to retrieve imaging settings for specified

video source from the Device.

GET IMAGING SETTINGS

Test Label: Get Imaging Settings - Get Imaging Settings

Test Case ID: GETIMAGINGSETTINGS-1

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Get Imaging Settings

Test Purpose: To verify that imaging settings for Device is received by Client using the GetImagingSettings operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with GetImagingSettings operation present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

2. Device responses with code HTTP 200 OK and GetImagingSettingsResponse message.

Test Result:

PASS -

- Client GetImagingSettings request messages are valid according to XML Schemas listed in Namespaces AND

- Client GetImagingSettings request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:GetImagingSettings AND

- Device response on the GetImagingSettings request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:GetImagingSettingsResponse.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_imaging_settings.get_imaging_settings

Imaging Settings Configuration Test Cases

Validated Feature: set_imaging_settings

Profile A Requirement: None

Profile C Requirement: None

Profile G Requirement: None

Profile Q Requirement: None

Profile S Requirement: None

Expected Scenarios Under Test:

- Client connects to Device to change imaging settings.

- Client is considered as supporting Imaging Settings Configuration if the following conditions are met:

  - Client is able to retrieve a imaging options using GetOptions operation AND

  - Client is able to change a imaging settings using SetImagingSettings operation.

- Client is considered as NOT supporting Imaging Settings Configuration if ANY of the following is TRUE:

with

GetOptions

operation present.

•

Device supports Imaging Service.

No valid responses for GetOptions request OR

  - No valid responses for SetImagingSettings request OR

  - SetImagingSettings 	request 	contains 	settings 	which 	does 	not 	correspong 	to GetOptionsResponse message for the same video source token.

GET OPTIONS

Test Label: Get Imaging Settings - Get Options

Test Case ID: SETIMAGINGSETTINGS-1

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Get Options

Test Purpose: To verify that imaging options for Device is received by Client using the GetOptions operation.

Pre-Requisite:

• The Network Trace Capture files contains at least one Conversation between Client and Device

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetOptions request message to retrieve imaging options for specified video source from the Device.

- Device responses with code HTTP 200 OK and GetOptionsResponse message.

Test Result:

PASS -

- Client GetOptions request messages are valid according to XML Schemas listed in Namespaces

AND

- Client GetOptions request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:GetOptions AND

- Device response on the GetOptions request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:GetOptionsResponse.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_imaging_settings.get_options

2.

Device responses with code HTTP 200 OK and

GetOptionsResponse

message.

3.

Client invokes

SetImagingSettings

request message to change imaging settings for specified

video source which are correspond to the resieved options on the Device.

SET IMAGING SETTINGS

Test Label: Set Imaging Settings - Set Imaging Settings

Test Case ID: SETIMAGINGSETTINGS-2

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Set Imaging Settings

Test Purpose: To verify that Client is able to change imaging settings on Device using the SetImagingSettings operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with SetImagingSettings operation present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

1. Client invokes GetOptions request message to retrieve imaging options for specified video source from the Device.

4. Device responses with code HTTP 200 OK and SetImagingSettingsResponse message.

Test Result:

PASS -

- Client SetImagingSettings request messages are valid according to XML Schemas listed in Namespaces AND

- Client SetImagingSettings request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:SetImagingSettings AND

- Device response on the SetImagingSettings request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:SetImagingSettingsResponse AND

- There is a Client GetOptions request in Test Procedure fulfills the following requirements:

- [S4] It invoked for the same Device as for the Client SetImagingSettings request AND

- [S5] It invoked before the Client SetImagingSettings request AND

- [S6] timg:VideoSourceToken element value is equal to timg:VideoSourceToken element from the SetImagingSettings request AND

- Device response on the GetOptions request fulfills the following requirements:

- [S7] It has HTTP 200 response code AND

- Settings from the SetImagingSettings request corresponds options recieved in the GetOptionsResponse message:

- [S8] IF the SetImagingSettings request contains tt:BacklightCompensation element THEN: • The GetOptionsResponse message contains tt:BacklightCompensation element AND

- The GetOptionsResponse message contains tt:BacklightCompensation/tt:Mode element value equal to tt:BacklightCompensation/tt:Mode element value from the SetImagingSettings request AND

- [S9] IF the SetImagingSettings request contains tt:BacklightCompensation/tt:Level element THEN:

- The GetOptionsResponse message contains tt:BacklightCompensation/tt:Level element AND

- tt:BacklightCompensation/tt:Level element value from the SetImagingSettings request is less or equal to tt:BacklightCompensation/tt:Level/tt:Max from the the GetOptionsResponse message AND

- tt:BacklightCompensation/tt:Level element value from the SetImagingSettings request is greater or equal to tt:BacklightCompensation/tt:Level/tt:Min from the the GetOptionsResponse message AND

- [S10] IF the SetImagingSettings request contains tt:Brightness element THEN:

- The GetOptionsResponse message contains tt:Brightness element AND

- tt:Brightness element value from the SetImagingSettings request is less or equal to tt:Brightness/tt:Max from the the GetOptionsResponse message AND

- tt:Brightness element value from the SetImagingSettings request is greater or equal to tt:Brightness/tt:Min from the the GetOptionsResponse message AND

- [S11] IF the SetImagingSettings request contains tt:ColorSaturation element THEN:

- The GetOptionsResponse message contains tt:ColorSaturation element AND

- tt:ColorSaturation element value from the SetImagingSettings request is less or equal to tt:ColorSaturation/tt:Max from the the GetOptionsResponse message AND

- tt:ColorSaturation element value from the SetImagingSettings request is greater or equal to tt:ColorSaturation/tt:Min from the the GetOptionsResponse message AND

- [S12] IF the SetImagingSettings request contains tt:Contrast element THEN:

- The GetOptionsResponse message contains tt:Contrast element AND

- tt:Contrast element value from the SetImagingSettings request is less or equal to tt:Contrast/tt:Max from the the GetOptionsResponse message AND

- tt:Contrast element value from the SetImagingSettings request is greater or equal to tt:Contrast/tt:Min from the the GetOptionsResponse message AND

- [S13] IF the SetImagingSettings request contains tt:Exposure element THEN:

- The GetOptionsResponse message contains tt:Exposure element AND

- The GetOptionsResponse message contains tt:Exposure/tt:Mode element value equal to tt:Exposure/tt:Mode element value from the SetImagingSettings request AND

- [S14] IF the SetImagingSettings request contains tt:Exposure/tt:Priority element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:Priority element AND

- The GetOptionsResponse message contains tt:Exposure/tt:Priority element value equal to tt:Exposure/tt:Priority element value from the SetImagingSettings request AND

- [S15] IF the SetImagingSettings request contains tt:Exposure/tt:MinExposureTime element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MinExposureTime element

AND

- tt:Exposure/tt:MinExposureTime element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MinExposureTime/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:MinExposureTime element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MinExposureTime/tt:Min from the the GetOptionsResponse message AND

- [S16] IF the SetImagingSettings request contains tt:Exposure/tt:MaxExposureTime element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MaxExposureTime element

AND

- tt:Exposure/tt:MaxExposureTime element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MaxExposureTime/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:MaxExposureTime element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MaxExposureTime/tt:Min from the the GetOptionsResponse message AND

- [S17] IF the SetImagingSettings request contains tt:Exposure/tt:MinGain element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MinGain element AND

- tt:Exposure/tt:MinGain element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MinGain/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:MinGain element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MinGain/tt:Min from the the GetOptionsResponse message AND

- [S18] IF the SetImagingSettings request contains tt:Exposure/tt:MaxGain element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MaxGain element AND

- tt:Exposure/tt:MaxGain element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MaxGain/tt:Max from the the GetOptionsResponse message

AND

- tt:Exposure/tt:MaxGain element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MaxGain/tt:Min from the the GetOptionsResponse message AND

- [S19] IF the SetImagingSettings request contains tt:Exposure/tt:MinIris element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MinIris element AND

- tt:Exposure/tt:MinIris element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MinIris/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:MinIris element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MinIris/tt:Min from the the GetOptionsResponse message AND

- [S20] IF the SetImagingSettings request contains tt:Exposure/tt:MaxIris element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:MaxIris element AND

- tt:Exposure/tt:MaxIris element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:MaxIris/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:MaxIris element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:MaxIris/tt:Min from the the GetOptionsResponse message AND

- [S21] IF the SetImagingSettings request contains tt:Exposure/tt:ExposureTime element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:ExposureTime element AND

- tt:Exposure/tt:ExposureTime element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:ExposureTime/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:ExposureTime element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:ExposureTime/tt:Min from the the GetOptionsResponse message AND

- [S22] IF the SetImagingSettings request contains tt:Exposure/tt:Gain element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:Gain element AND

- tt:Exposure/tt:Gain element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:Gain/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:Gain element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:Gain/tt:Min from the the GetOptionsResponse message AND

- [S23] IF the SetImagingSettings request contains tt:Exposure/tt:Iris element THEN:

- The GetOptionsResponse message contains tt:Exposure/tt:Iris element AND

- tt:Exposure/tt:Iris element value from the SetImagingSettings request is less or equal to tt:Exposure/tt:Iris/tt:Max from the the GetOptionsResponse message AND

- tt:Exposure/tt:Iris element value from the SetImagingSettings request is greater or equal to tt:Exposure/tt:Iris/tt:Min from the the GetOptionsResponse message AND

- [S24] IF the SetImagingSettings request contains tt:Focus element THEN:

- The GetOptionsResponse message contains tt:Focus element AND

- The GetOptionsResponse message contains tt:Focus/tt:AutoFocusModes element value equal to tt:Focus/tt:AutoFocusMode element value from the SetImagingSettings request AND

- [S25] IF the SetImagingSettings request contains tt:Focus/tt:DefaultSpeed element THEN: • The GetOptionsResponse message contains tt:Focus/tt:DefaultSpeed element AND

- tt:Focus/tt:DefaultSpeed element value from the SetImagingSettings request is less or equal to tt:Focus/tt:DefaultSpeed/tt:Max from the the GetOptionsResponse message

AND

- tt:Focus/tt:DefaultSpeed element value from the SetImagingSettings request is greater or equal to tt:Focus/tt:DefaultSpeed/tt:Min from the the GetOptionsResponse message

AND

- [S26] IF the SetImagingSettings request contains tt:Focus/tt:NearLimit element THEN:

- The GetOptionsResponse message contains tt:Focus/tt:NearLimit element AND

- tt:Focus/tt:NearLimit element value from the SetImagingSettings request is less or equal to tt:Focus/tt:NearLimit/tt:Max from the the GetOptionsResponse message AND

- tt:Focus/tt:NearLimit element value from the SetImagingSettings request is greater or equal to tt:Focus/tt:NearLimit/tt:Min from the the GetOptionsResponse message AND

- [S27] IF the SetImagingSettings request contains tt:Focus/tt:FarLimit element THEN:

- The GetOptionsResponse message contains tt:Focus/tt:FarLimit element AND

- tt:Focus/tt:FarLimit element value from the SetImagingSettings request is less or equal to tt:Focus/tt:FarLimit/tt:Max from the the GetOptionsResponse message AND

- tt:Focus/tt:FarLimit element value from the SetImagingSettings request is greater or equal to tt:Focus/tt:FarLimit/tt:Min from the the GetOptionsResponse message AND

- [S28] IF the SetImagingSettings request contains tt:IrCutFilter element THEN:

- The GetOptionsResponse message contains tt:IrCutFilterModes element AND

- The GetOptionsResponse message contains tt:IrCutFilterModes element value equal to tt:IrCutFilter element value from the SetImagingSettings request AND

- [S29] IF the SetImagingSettings request contains tt:Sharpness element THEN:

- The GetOptionsResponse message contains tt:Sharpness element AND

- tt:Sharpness element value from the SetImagingSettings request is less or equal to tt:Sharpness/tt:Max from the the GetOptionsResponse message AND

- tt:Sharpness element value from the SetImagingSettings request is greater or equal to tt:Sharpness/tt:Min from the the GetOptionsResponse message AND

- [S30] IF the SetImagingSettings request contains tt:WideDynamicRange element THEN:

- The GetOptionsResponse message contains tt:WideDynamicRange element AND

- The GetOptionsResponse message contains tt:WideDynamicRange/tt:Mode element value equal to tt:WideDynamicRange/tt:Mode element value from the SetImagingSettings request AND

- [S31] IF the SetImagingSettings request contains tt:WideDynamicRange/tt:Level element THEN:

- The GetOptionsResponse message contains tt:WideDynamicRange/tt:Level element

AND

- tt:WideDynamicRange/tt:Level element value from the SetImagingSettings request is less or equal to tt:WideDynamicRange/tt:Level/tt:Max from the the GetOptionsResponse message AND

- tt:WideDynamicRange/tt:Level element value from the SetImagingSettings request is greater or equal to tt:WideDynamicRange/tt:Level/tt:Min from the the GetOptionsResponse message AND

- [S32] IF the SetImagingSettings request contains tt:WhiteBalance element THEN:

- The GetOptionsResponse message contains tt:WhiteBalance element AND

- The GetOptionsResponse message contains tt:WhiteBalance/tt:Mode element value equal to tt:WhiteBalance/tt:Mode element value from the SetImagingSettings request

AND

- [S33] IF the SetImagingSettings request contains tt:WhiteBalance/tt:CrGain element THEN:

•

The

GetOptionsResponse

message contains

tt:Extension/tt:ImageStabilization/

tt:Level

element AND

•

tt:Extension/tt:ImageStabilization/tt:Level

element value from the

SetImagingSettings

request is less or equal to

tt:Extension/tt:ImageStabilization/tt:Level/tt:Max

from the the

GetOptionsResponse

message AND

The GetOptionsResponse message contains tt:WhiteBalance/tt:YrGain element AND

- tt:WhiteBalance/tt:CrGain element value from the SetImagingSettings request is less or equal to tt:WhiteBalance/tt:YrGain/tt:Max from the the GetOptionsResponse message

AND

- tt:WhiteBalance/tt:CrGain element value from the SetImagingSettings request is greater or equal to tt:WhiteBalance/tt:YrGain/tt:Min from the the GetOptionsResponse message

AND

- [S34] IF the SetImagingSettings request contains tt:WhiteBalance/tt:CbGain element THEN:

- The GetOptionsResponse message contains tt:WhiteBalance/tt:YbGain element AND

- tt:WhiteBalance/tt:CbGain element value from the SetImagingSettings request is less or equal to tt:WhiteBalance/tt:YbGain/tt:Max from the the GetOptionsResponse message

AND

- tt:WhiteBalance/tt:CbGain element value from the SetImagingSettings request is greater or equal to tt:WhiteBalance/tt:YbGain/tt:Min from the the GetOptionsResponse message AND

- [S35] IF the SetImagingSettings request contains tt:Extension/tt:ImageStabilization element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:ImageStabilization element AND

- The GetOptionsResponse message contains tt:Extension/tt:ImageStabilization/ tt:Mode element value equal to tt:Extension/tt:ImageStabilization/tt:Mode element value from the SetImagingSettings request AND

- [S36] IF the SetImagingSettings request contains tt:Extension/tt:ImageStabilization/ tt:Level element THEN:

- tt:Extension/tt:ImageStabilization/tt:Level element value from the SetImagingSettings request is greater or equal to tt:Extension/tt:ImageStabilization/tt:Level/tt:Min from the the GetOptionsResponse message AND

- [S37] IF the SetImagingSettings request contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment element AND

- The GetOptionsResponse message contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment/tt:BoundaryType element value equal to each tt:Extension/tt:Extension/tt:IrCutFilterAutoAdjustment/tt:BoundaryType element value from the SetImagingSettings request AND

- [S38] IF the SetImagingSettings request contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment/tt:BoundaryOffset element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment/tt:BoundaryOffset element with value equal to true AND

- tt:Extension/tt:Extension/tt:IrCutFilterAutoAdjustment/tt:BoundaryOffset element value from the SetImagingSettings request is less or equal to +1.0 AND

- tt:Extension/tt:Extension/tt:IrCutFilterAutoAdjustment/tt:BoundaryOffset element value from the SetImagingSettings request is greater or equal to -1.0 AND

- [S39] IF the SetImagingSettings request contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment/tt:ResponseTime element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/ tt:IrCutFilterAutoAdjustment/tt:ResponseTimeRange element AND

- tt:Extension/tt:Extension/tt:IrCutFilterAutoAdjustment/tt:ResponseTime element value from the SetImagingSettings request is less or equal to tt:Extension/ tt:Extension/tt:IrCutFilterAutoAdjustment/tt:ResponseTimeRange from the the GetOptionsResponse message AND

- tt:Extension/tt:Extension/tt:IrCutFilterAutoAdjustment/tt:ResponseTime element value from the SetImagingSettings request is greater or equal to tt:Extension/ tt:Extension/tt:IrCutFilterAutoAdjustment/tt:ResponseTimeRange from the the GetOptionsResponse message AND

- [S40] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:ToneCompensation element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/tt:Extension/ tt:ToneCompensationOptions element AND

- The GetOptionsResponse message contains tt:Extension/tt:Extension/tt:Extension/ tt:ToneCompensationOptions/tt:Mode element value equal to tt:Extension/ tt:Extension/tt:Extension/tt:ToneCompensation/tt:Mode element value from the SetImagingSettings request AND

- [S41] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:ToneCompensation/tt:Level element THEN:

- The GetOptionsResponse message contains >tt:Extension/tt:Extension/tt:Extension/ tt:ToneCompensationOptions/tt:Level element with value equal to true AND

- tt:Extension/tt:Extension/tt:Extension/tt:ToneCompensation/tt:Level element value from the SetImagingSettings request is less or equal to +1.0 AND

- tt:Extension/tt:Extension/tt:Extension/tt:ToneCompensation/tt:Level element value from the SetImagingSettings request is greater or equal to 0.0 AND

- [S42] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:Defogging element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/tt:Extension/ tt:DefoggingOptions element AND

- The GetOptionsResponse message contains tt:Extension/tt:Extension/tt:Extension/ tt:DefoggingOptions/tt:Mode element value equal to tt:Extension/tt:Extension/ tt:Extension/tt:Defogging/tt:Mode element value from the SetImagingSettings request

AND

- [S43] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:Defogging/tt:Level element THEN:

- The GetOptionsResponse message contains >tt:Extension/tt:Extension/tt:Extension/ tt:DefoggingOptions/tt:Level element with value equal to true AND

- tt:Extension/tt:Extension/tt:Extension/tt:Defogging/tt:Level element value from the SetImagingSettings request is less or equal to +1.0 AND

- tt:Extension/tt:Extension/tt:Extension/tt:Defogging/tt:Level element value from the SetImagingSettings request is greater or equal to 0.0 AND

- [S44] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:NoiseReduction element THEN:

- The GetOptionsResponse message contains tt:Extension/tt:Extension/tt:Extension/ tt:NoiseReductionOptions element AND

- [S45] IF the SetImagingSettings request contains tt:Extension/tt:Extension/tt:Extension/ tt:NoiseReduction/tt:Level element THEN:

- The GetOptionsResponse message contains >tt:Extension/tt:Extension/tt:Extension/ tt:NoiseReductionOptions/tt:Level element with value equal to true AND

- tt:Extension/tt:Extension/tt:Extension/tt:NoiseReduction/tt:Level element value from the SetImagingSettings request is less or equal to +1.0 AND

- tt:Extension/tt:Extension/tt:Extension/tt:NoiseReduction/tt:Level element value from the SetImagingSettings request is greater or equal to 0.0.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_imaging_settings.get_imaging_settings

Focus Move Capabilities Test Cases

Validated Feature: get_move_options

Profile A Requirement: None

Profile C Requirement: None

Profile G Requirement: None

Profile Q Requirement: None

Profile S Requirement: None

Expected Scenarios Under Test:

- Client connects to Device to get focus move capabilities.

- Client is considered as supporting Focus Move Capabilities if the following conditions are met:

  - Client is able to retrieve a focus move options using GetMoveOptions operation AND

- Client is considered as NOT supporting Focus Move Capabilities if ANY of the following is TRUE:

  - No valid responses for GetMoveOptions request OR

1.

Client invokes

GetMoveOptions

request message to retrieve focus move options for specified

video source from the Device.

GET FOCUS MOVE OPTIONS

Test Label: Get Move Options

Test Case ID: GETMOVEOPTIONS-1

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Get Move Options

Test Purpose: To verify that Client is able retrive focus move capabilities from Device using the GetMoveOptions operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with GetMoveOptions operation present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

2. Device responses with code HTTP 200 OK and GetMoveOptionsResponse message.

Test Result:

PASS -

- Client GetMoveOptions request messages are valid according to XML Schemas listed in Namespaces AND

- Client GetMoveOptions request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:GetMoveOptions AND

- Device response on the GetMoveOptions request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:GetMoveOptionsResponse.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: get_move_options.get_move_options

Focus Control Test Cases

Validated Feature: focus_control

Profile A Requirement: None

Profile C Requirement: None

Profile G Requirement: None

Profile Q Requirement: None

Profile S Requirement: None

Expected Scenarios Under Test:

- Client connects to Device to control focus.

- Client is considered as supporting Focus Control if the following conditions are met:

  - Client supports get_move_options feature AND

  - Client is able to invoke Absolute OR Relative OR Continuous focus move using Move operation AND

Profile S Reference:

None

Feature Under Test:

Absolute Focus Move

If Client is able to invoke Continuous focus move Client is able to invoke stop focus move using Stop operation.

- Client is considered as NOT supporting Focus Control if ANY of the following is TRUE:

  - Client does not support get_move_options feature OR

  - No valid responses for Move request OR

  - Move request contains settings which does not correspong to GetMoveOptions message for the same video source token OR

  - No valid responses for Stop request if Stop request is supported by the Client OR

  - Stop request is not supported, in the case Continuous focus move is supported by the Client.

ABSOLUTE FOCUS MOVE

Test Label: Focus Control - Absolute Focus Move

Test Case ID: FOCUSCONTROL-1

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Test Purpose: To verify that Client is able retrive absolute focus move on Device using the Move operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with Move operation with tt:Absolute element present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetMoveOptions request message to retrieve focus move options for specified video source from the Device.

- Device responses with code HTTP 200 OK and GetMoveOptionsResponse message.

- If GetMoveOptionsResponse message contains tt:Absolute element Client invokes Move request message for specified video source with tt:Absolute element with parameters which are correspond to the resieved focus move options to start absolute focus movement on the Device.

- Device responses with code HTTP 200 OK and MoveResponse message.

Test Result:

PASS -

- Client Move request messages are valid according to XML Schemas listed in Namespaces AND

- Client Move request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:Move AND

- [S2] It contains timg:Focus/tt:Absolute element AND

- Device response on the Move request fulfills the following requirements:

- [S3] It has HTTP 200 response code AND

- [S4] soapenv:Body element has child element timg:MoveResponse AND

- There is a Client GetMoveOptions request in Test Procedure fulfills the following requirements:

- [S5] It invoked for the same Device as for the Client Move request AND

- [S6] It invoked before the Client Move request AND

- [S7] timg:VideoSourceToken element value is equal to timg:VideoSourceToken element from the Move request AND

- Device response on the GetMoveOptions request fulfills the following requirements:

- [S8] It has HTTP 200 response code AND

- [S9] soapenv:Body element has child element timg:GetMoveOptionsResponse AND

- [S10] It contains timg:MoveOptions\tt:Absolute element AND

- Settings from the Move request corresponds options recieved in the GetMoveOptionsResponse message:

- [S11] timg:Focus/tt:Absolute/tt:Position element value from the Move request is less or equal to timg:MoveOptions/tt:Absolute/tt:Position/tt:Max from the the GetMoveOptionsResponse message AND

- [S12] timg:Focus/tt:Absolute/tt:Position element value from the Move request is greater or equal to timg:MoveOptions/tt:Absolute/tt:Position/tt:Min from the the GetMoveOptionsResponse message AND

- [S13] IF the Move request contains timg:Focus/tt:Absolute/tt:Speed element THEN:

- The GetMoveOptionsResponse message contains timg:MoveOptions/tt:Absolute/ tt:Speed element AND

- timg:Focus/tt:Absolute/tt:Speed 	element 	value 	from 	the 	Move 	request 	is less or equal to timg:MoveOptions/tt:Absolute/tt:Speed/tt:Max from the the GetMoveOptionsResponse message AND

- timg:Focus/tt:Absolute/tt:Speed 	element 	value 	from 	the 	Move 	request 	is greater or equal to timg:MoveOptions/tt:Absolute/tt:Speed/tt:Min from the the GetMoveOptionsResponse message.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: focus_control.absolute_focus_move

RELATIVE FOCUS MOVE

Test Label: Focus Control - Relative Focus Move

Test Case ID: FOCUSCONTROL-2

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

PASS -

•

Client

Move

request messages are valid according to XML Schemas listed in

Namespaces

AND

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Relative Focus Move

Test Purpose: To verify that Client is able retrive relative focus move on Device using the Move operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with Move operation with tt:Relative element present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetMoveOptions request message to retrieve focus move options for specified video source from the Device.

- Device responses with code HTTP 200 OK and GetMoveOptionsResponse message.

- If GetMoveOptionsResponse message contains tt:Relative element Client invokes Move request message for specified video source with tt:Relative element with parameters which are correspond to the resieved focus move options to start relative focus movement on the Device.

- Device responses with code HTTP 200 OK and MoveResponse message.

Test Result:

- Client Move request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:Move AND

- [S2] It contains timg:Focus/tt:Relative element AND

- Device response on the Move request fulfills the following requirements:

- [S3] It has HTTP 200 response code AND

- [S4] soapenv:Body element has child element timg:MoveResponse AND

- There is a Client GetMoveOptions request in Test Procedure fulfills the following requirements:

- [S5] It invoked for the same Device as for the Client Move request AND

- [S6] It invoked before the Client Move request AND

- [S7] timg:VideoSourceToken element value is equal to timg:VideoSourceToken element from the Move request AND

- Device response on the GetMoveOptions request fulfills the following requirements:

- [S8] It has HTTP 200 response code AND

- [S9] soapenv:Body element has child element timg:GetMoveOptionsResponse AND

- [S10] It contains timg:MoveOptions\tt:Relative element AND

- Settings 	from 	the 	Move 	request 	corresponds 	options 	recieved 	in 	the GetMoveOptionsResponse message:

- [S11] timg:Focus/tt:Relative/tt:Distance element value from the Move request is less or equal to timg:MoveOptions/tt:Relative/tt:Distance/tt:Max from the the GetMoveOptionsResponse message AND

Profile Q Reference:

None

Profile S Reference:

None

[S12] timg:Focus/tt:Relative/tt:Distance element value from the Move request is greater or equal to timg:MoveOptions/tt:Relative/tt:Distance/tt:Min from the the GetMoveOptionsResponse message AND

- [S13] IF the Move request contains timg:Focus/tt:Relative/tt:Speed element THEN:

- The GetMoveOptionsResponse message contains timg:MoveOptions/tt:Relative/ tt:Speed element AND

- timg:Focus/tt:Relative/tt:Speed element value from the Move request is less or equal to timg:MoveOptions/tt:Relative/tt:Speed/tt:Max from the the GetMoveOptionsResponse message AND

- timg:Focus/tt:Relative/tt:Speed 	element 	value 	from 	the 	Move 	request 	is greater or equal to timg:MoveOptions/tt:Relative/tt:Speed/tt:Min from the the GetMoveOptionsResponse message.

FAIL -

- The Client failed PASS criteria.

Validated Feature List: focus_control.relative_focus_move

CONTINUOUS FOCUS MOVE

Test Label: Focus Control - Continuous Focus Move

Test Case ID: FOCUSCONTROL-3

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Feature Under Test: Continuous Focus Move

Test Purpose: To verify that Client is able retrive continuous focus move on Device using the Move operation.

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with Move operation with tt:Continuous element present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes GetMoveOptions request message to retrieve focus move options for specified video source from the Device.

- Device responses with code HTTP 200 OK and GetMoveOptionsResponse message.

- If GetMoveOptionsResponse message contains tt:Continuous element Client invokes Move request message for specified video source with tt:Relative element with parameters which are correspond to the resieved focus move options to start continuous focus movement on the Device.

- Device responses with code HTTP 200 OK and MoveResponse message.

•

[

S11]

timg:Focus/tt:Continuous/tt:Speed

element value from the

Move

request

is less or equal to

timg:MoveOptions/tt:Continuous/tt:Speed/tt:Max

from the the

GetMoveOptionsResponse

message AND

•

[

S12]

timg:Focus/tt:Continuous/tt:Speed

element value from the

Move

request is

greater or equal to

timg:MoveOptions/tt:Continuous/tt:Speed/tt:Min

from the the

GetMoveOptionsResponse

message.

Test Result:

PASS -

- Client Move request messages are valid according to XML Schemas listed in Namespaces AND

- Client Move request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:Move AND

- [S2] It contains timg:Focus/tt:Continuous element AND

- Device response on the Move request fulfills the following requirements:

- [S3] It has HTTP 200 response code AND

- [S4] soapenv:Body element has child element timg:MoveResponse AND

- There is a Client GetMoveOptions request in Test Procedure fulfills the following requirements:

- [S5] It invoked for the same Device as for the Client Move request AND

- [S6] It invoked before the Client Move request AND

- [S7] timg:VideoSourceToken element value is equal to timg:VideoSourceToken element from the Move request AND

- Device response on the GetMoveOptions request fulfills the following requirements:

- [S8] It has HTTP 200 response code AND

- [S9] soapenv:Body element has child element timg:GetMoveOptionsResponse AND

- [S10] It contains timg:MoveOptions\tt:Continuous element AND

- Settings 	from 	the 	Move 	request 	corresponds 	options 	recieved 	in 	the GetMoveOptionsResponse message:

FAIL -

- The Client failed PASS criteria.

Validated Feature List: focus_control.continuous_focus_move

STOP

Test Label: Focus Control - Stop

Test Case ID: FOCUSCONTROL-4

Profile A Reference: None

Profile C Reference: None

Profile G Reference: None

Profile Q Reference: None

Profile S Reference: None

Feature Under Test: Stop

Test Purpose: To verify that Client is able retrivefocus move options fromon Device using the Stop operation.

•

The Client failed PASS criteria.

Validated Feature List:

focus_control.stop

Pre-Requisite:

- The Network Trace Capture files contains at least one Conversation between Client and Device with Stop operation present.

- Device supports Imaging Service.

Test Procedure (expected to be reflected in network trace file):

- Client invokes Stop request message to stop focus move for specified video source for the Device.

- Device responses with code HTTP 200 OK and StopResponse message.

Test Result:

PASS -

- Client Stop request messages are valid according to XML Schemas listed in Namespaces AND

- Client Stop request in Test Procedure fulfills the following requirements:

- [S1] soapenv:Body element has child element timg:Stop AND

- Device response on the Stop request fulfills the following requirements:

- [S2] It has HTTP 200 response code AND

- [S3] soapenv:Body element has child element timg:StopResponse.

FAIL -

ONVIF Imaging Client Test Specification Version 16.07

- Appendix A. Revision History

December 21, 2016 Version 16.01

- Focus Control Test Cases was splitted on two parts (Focus Move Capabilities Test Cases and Focus Control Test Cases) to handle Device capabilities issue.

December 18, 2016 Version 16.01

- Get Imaging Capabilities Test Cases feature definition was updated

October 20, 2016 Version 15.10

- Initial version:

General parts added

Get Imaging Capabilities Test Cases added

Video Sources List Test Cases added

Get Imaging Settings Test Cases added

Imaging Settings Configuration Test Cases added

Focus Control Test Cases added